IO LIST PLC

WTP 10 CMD

DOCUMENT REVISION RECORD

| Date | Revision | Description |
| --- | --- | --- |
| 02 January 2026 | 00 | First Issued |

Contents

DIGITAL INPUT MODULES	4

DIGITAL OUTPUT MODULES	5

ANALOG INPUT MODULES	6

| No | Tag | Description | Location |  |  | I/O |  | PLC |  |  | Range |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | From | To | Service | Signal | Type | Module | Channel | Address | HH | H | SP | L | LL |  |
| # DIGITAL INPUT MODULES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EMG | Emergency Stop | MCP | MCC | HMI | DI | Dry | C1 |  | I.0 | - | - | - | - | - | - |
|  | T-TP-01A | TP-01A | MCP | MCC | HMI | DI | Dry |  |  | I.1 | - | - | - | - | - | - |
|  | T-TP-01B | TP-01B | MCP | MCC | HMI | DI | Dry |  |  | I.2 | - | - | - | - | - | - |
|  | T-TP-XXA | Trip TP-XXA | MCP | MCC | HMI | DI | Dry |  |  | I.3 | - | - | - | - | - | - |
|  | T-TP-XXB | Trip TP-XXB | MCP | MCC | HMI | DI | Dry |  |  | I.4 | - | - | - | - | - | - |
|  | LS-101 | LS-101 | MCP | MCC | HMI | DI | Dry |  |  | I.5 | - | - | - | - | - | - |
|  | AV-101 | AV-101 | MCP | MCC | HMI | DI | Dry |  |  | I.6 | - | - | - | - | - | - |
|  | AV-102 | AV-102 | MCP | MCC | HMI | DI | Dry |  |  | I.7 | - | - | - | - | - | - |
|  | AV-103 | AV-103 | MCP | MCC | HMI | DI | Dry |  |  | I.8 | - | - | - | - | - | - |
|  | AV-104 | AV-104 | MCP | MCC | HMI | DI | Dry |  |  | I.9 | - | - | - | - | - | - |
|  | AV-105 | AV-105 | MCP | MCC | HMI | DI | Dry |  |  | I.10 | - | - | - | - | - | - |
|  | AV-106 | AV-106 | MCP | MCC | HMI | DI | Dry |  |  | I.11 | - | - | - | - | - | - |
|  | AV == VES? VES 107 s/d VES 123? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LS–0104 Threated water tank level switch |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # DIGITAL OUTPUT MODULES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 1 | QTP-01A | TP-01A | MCP | MCC | Field | DO | R1 | C1 | 0 | O.0 | - | - | - | - | - | - |
| - 2 | QTP-01B | TP-01B | MCP | MCC | Field | DO | R2 |  | 1 | O.1 | - | - | - | - | - | - |
| - 3 | QTP-XXA | TP-XXA | MCP | MCC | Field | DO | R3 |  | 2 | O.2 | - | - | - | - | - | - |
| - 4 | QTP-XXB | TP-XXB | MCP | MCC | Field | DO | R4 |  | 3 | O.3 | - | - | - | - | - | - |
| - 5 | QDP-01 | DP-01 | MCP | MCC | Field | DO | R5 |  | 4 | O.4 | - | - | - | - | - | - |
| - 6 | QAV-101 | AV-101 | MCP | MCC | Field | DO | R6 |  | 5 | O.5 | - | - | - | - | - | - |
| - 7 | QAV-102 | AV-102 | MCP | MCC | Field | DO | R7 |  | 6 | O.6 | - | - | - | - | - | - |
| - 8 | QAV-103 | AV-103 | MCP | MCC | Field | DO | R8 |  | 7 | O.7 | - | - | - | - | - | - |
| - 9 | QAV-104 | AV-104 | MCP | MCC | Field | DO | R9 |  | 8 | O.8 | - | - | - | - | - | - |
| - 10 | QAV-105 | AV-105 | MCP | MCC | Field | DO | R10 |  | 9 | O.9 | - | - | - | - | - | - |
|  | QAV-106 | AV-106 | MCP | MCC | Field | DO | R11 | C2 | 0 | O.10 | - | - | - | - | - | - |
|  | QBUZZER | ALARM BUSSER | MCP | MCC | Field | DO | R12 |  | 1 | O.11 | - | - | - | - | - | - |
|  | AT01-AT04? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # ANALOG INPUT MODULES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | AIFT-101 | FT-101 | Field | MCP | HMI | AI | 4-20mA | C3 | 01 | A1 | SP+5% | SP+8% | SP | SP-30% | SP-20% |  |
| 2 | AIAE-101 | AE-101 | Field | MCP | HMI | AI | 4-20mA |  | 02 | A2 | SP+5% | SP+8% | SP | SP-30% | SP-20% |  |
| 3 | AIAE-102 | AE-102 | Field | MCP | HMI | AI | 4-20mA |  | 03 | A3 | SP+5% | SP+8% | SP | SP-30% | SP-20% |  |
| 4 |  |  | Field | MCP | HMI | AI | 4-20mA |  | 04 |  | SP+5% | SP+8% | SP | SP-30% | SP-20% |  |